通过TCP协议发送指令数据。

获取参数

- 心跳包（读取常规信息）

发送 “TOCK”

返回信息：如下结构体，以字节流的形式由下位机发送给上位机

public struct PMC400_STATUS

{

public Int32 PMC400_X_StartSpeed;

public Int32 PMC400_Y_StartSpeed;

public Int32 PMC400_Z_StartSpeed;

public Int32 PMC400_U_StartSpeed;//初速度

public Int32 PMC400_X_AccSpeed;

public Int32 PMC400_Y_AccSpeed;

public Int32 PMC400_Z_AccSpeed;

public Int32 PMC400_U_AccSpeed;//加速度

public Int32 PMC400_X_FinalSpeed;

public Int32 PMC400_Y_FinalSpeed;

public Int32 PMC400_Z_FinalSpeed;

public Int32 PMC400_U_FinalSpeed;//终速度

public Int32 PMC400_X_CurentPosition;

public Int32 PMC400_Y_CurentPosition;

public Int32 PMC400_Z_CurentPosition;

public Int32 PMC400_U_CurentPosition;//当前位置（输出）

public Int32 PMC400_X_PositionEncode;

public Int32 PMC400_Y_PositionEncode;

public Int32 PMC400_Z_PositionEncode;

public Int32 PMC400_U_PositionEncode;//反馈位置（光栅尺反馈）

public Int16 PMC400_X_Is_MoveCommand;

public Int16 PMC400_Y_Is_MoveCommand;

public Int16 PMC400_Z_Is_MoveCommand;

public Int16 PMC400_U_Is_MoveCommand;//各轴的宏观运动状态： 正向(10)，负向(11)，回0(12)，运动完成(13)，无状态(14)

public Int16 PMC400_IO_IN;//IO输入口（8位）

public Int16 PMC400_IO_OUT;//IO输出口（8位）

public Int16 PMC400_X_Is_Moveing;

public Int16 PMC400_Y_Is_Moveing;

public Int16 PMC400_Z_Is_Moveing;

public Int16 PMC400_U_Is_Moveing;// 各轴是否正在运行，1为停止

public Int32 PMC400_X_Pulse_Remaining;

public Int32 PMC400_Y_Pulse_Remaining;

public Int32 PMC400_Z_Pulse_Remaining;

public Int32 PMC400_U_Pulse_Remaining;// 各轴当前位置：待发出脉冲

public Int16 PMC400_X_MicroSteps;

public Int16 PMC400_Y_MicroSteps;

public Int16 PMC400_Z_MicroSteps;

public Int16 PMC400_U_MicroSteps;// 各轴细分数

public Int32 PMC400_X_ScrewLead;

public Int32 PMC400_Y_ScrewLead;

public Int32 PMC400_Z_ScrewLead;

public Int32 PMC400_U_ScrewLead;// 各轴丝杆导程，单位：nm

public Int32 PMC400_X_SpeedRatio;

public Int32 PMC400_Y_SpeedRatio;

public Int32 PMC400_Z_SpeedRatio;

public Int32 PMC400_U_SpeedRatio;// 各轴转速比，光栅尺走1mm（或1°）需要丝杆转动的圈数, 此处为实际值的1000倍

public Int32 PMC400_X_Electric_Current;

public Int32 PMC400_Y_Electric_Current;

public Int32 PMC400_Z_Electric_Current;

public Int32 PMC400_U_Electric_Current;//各轴电流，暂无用

public Int32 PMC400_X_Resolution_Ratio;

public Int32 PMC400_Y_Resolution_Ratio;

public Int32 PMC400_Z_Resolution_Ratio;

public Int32 PMC400_U_Resolution_Ratio;// 各轴光栅尺分辨率，单位：脉冲数/毫米（脉冲数/度）

public Int32 PMC400_X_Dest_PositionEncode;

public Int32 PMC400_Y_Dest_PositionEncode;

public Int32 PMC400_Z_Dest_PositionEncode;

public Int32 PMC400_U_Dest_PositionEncode;// 各轴绝对位置

public Int32 PMC400_XYZU_EL_ORG; //包含零位,正负限位等状态, x轴: bit0-7; Y轴: bit8-15; Z轴: bit16-23; U轴: bit24-31

//8bit信息: 4bit-正限位; 5bit-负限位; 6bit-零位;

public Int32 PMC400_PROGRAM_LINE_NUM;// Flash里有具体的运动指令，在程序运动模式下，系统从flash取得指令，一条一条的执行

public Int32 PMC400_X_MoveDistance_Read;

public Int32 PMC400_Y_MoveDistance_Read;

public Int32 PMC400_Z_MoveDistance_Read;

public Int32 PMC400_U_MoveDistance_Read;// 读取到的各轴运动距离在这里

public Int32 PCL400_PROGRAM_LINE_NUM_JP;// 程序运动模式下， 指令的截止数目

public Int32 PMC400_SN; // 序列号

public Int32 PMC400_IP;// IP地址

public Int32 PMC400_NM;// 子网掩码

public Int32 PMC400_GW;// 网关

public Int32 PCL400_32_temp;//备用

public Int32 PCL400_CRC_SUM;// CRC校验, 包括指令的截止数目在内，结构体之前的所有数据的和。

};

设置参数，无返回值

- 初速度：

“COMMAND:STARTSPEED X”+速度（脉冲数/秒）+“；”

- 加速度

“COMMAND:ACCSPEED X”+加速度（脉冲数/秒方）+“；”

- 终速度

“COMMAND:FINALSPEED X”+速度（脉冲数/秒）+“；”

- 开环移动

“COMMAND:OPENMOVE X”+距离（脉冲）+“；”

- 闭环移动

“COMMAND:CLOSEMOVE X”+距离（脉冲）+“；”

- 开始执行

“COMMAND:START；”

- 停止

“COMMAND:STOP；”

- 设置IO输出口电平

“COMMAND:IOOUTPUT”+电平大小（二进制8位）+“；”

- 保存配置

“COMMAND:SAVE；”

- 轴的驱动器的细分数

“COMMAND:MICROSTEPS X”+细分数+“；”

- 轴的丝杆导程

“COMMAND:SCREWLEAD X”+长度（单位为纳米）+“；”

- 转速比

“COMMAND:SPEEDRATIO X”+转速比（实际值1000倍）+“；”

- 轴的限位逻辑(常开，常闭)

“COMMAND:LIMITTTL X”+常开0常闭1+“；”

- 指定轴的回零

“COMMAND:HOMEBACK X；”

- 轴的分辨率

“COMMAND:RESOLUTIONRATIO X”+分辨率（脉冲）+“；”

PID相关指令

typedef struct _ PID_CFG{

float kp[4];

float ki[4];

float kii[4]

float kd[4]; //32位 //各轴PID参数 [0-3] =[x,y,z,u]

float ratioKp[4];

float ratioKi[4];

float ratioKd[4]; //32位 //各轴PID参数变化比率, 是个百分比, [0-3] =[x,y,z,u]

float slewRate[4]; //32位 //积分摆动范围, [0-3] =[x,y,z,u]

signed char kiien[4]; //8位 [0-3] =[x,y,z,u]

signed char lastkiien[4]; //8位 //[0-3] =[x,y,z,u]

} PID_CFG; //PID参数结构

- 设置PID参数

PIDSET:[PID参数结构] //注意：指令中“:”与[]间不要有空格，“[”和“]”不要传，PID参数结构指 PID_CFG 的内存样式。

下位机返回：“PIDSETOK”。

- 读取PID参数

PIDGET

下位机返回: [PID参数结构]。

- 存储PID参数

PIDSTOR

下位机返回: "PIDSTOROK"。

- 设置/查询PID使能（使能指使用全闭环模式）

PIDONOFF: #1 #2 #3	//#1 =W或R, W =设置，R =读取;  #2 =ON 或 OFF; #3 =XYZU任意组合。

下位机返回：PIDONOFF: &1 &2 &3 &4 //&1, &2, &3, &4 =ON 或 OFF， 依次对应 XYZU轴参数。

W 表示设置，R 表示读取, ON 表示打开PID，OFF 表示关闭PID,

#1 =R时，#2参数任意，不会被设置。